MAES LAB 7 Alternative Code

// define the pin numbers

const int trigPin = 11;

const int echoPin = 12;

// define variables

long duration;

float distance, distanceinches, distanceThreshold;

void setup() {

Serial.begin(9600); // Starts the serial communication

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output

pinMode(echoPin, INPUT);  // Sets the echoPin as an Input

// LED pins

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

}

void loop() {

// Clears the trigPin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Sets the trigPin on HIGH state for 10 microseconds

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculating the distance in cm

distance = (duration / 2.0) * 1e-6 * 340.0 * 100.0;

// Convert to inches

distanceinches = distance / 2.54;

// Print the distance on the Serial Monitor

Serial.print("Distance = ");

Serial.print(distance);

Serial.print(" cm; ");

Serial.print("Distance = ");

Serial.print(distanceinches);

Serial.println(" inches");

// Set threshold distance to activate LEDs

distanceThreshold = 80;

// LED conditions based on distance

if (distance > distanceThreshold) {

digitalWrite(2, LOW);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

}

if (distance < distanceThreshold && distance > distanceThreshold - 30) {

digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

}

if (distance < distanceThreshold - 30 && distance > distanceThreshold - 50) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

}

if (distance < distanceThreshold - 50 && distance > distanceThreshold - 70) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

}

delay(200); // Wait for 200 milliseconds

}