MES Lab_9 Alternative Codes

int switchrotation = 2; // input pin to switch the direction of rotation

int in1 = 8;           // Declaring where our module is wired

int in2 = 9;

int ConA = 10;         // PWM pin

int speed1;

void setup() {

Serial.begin(9600);

pinMode(switchrotation, INPUT);

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(ConA, OUTPUT);

}

void TurnMotorA1() {

digitalWrite(in1, LOW);     // Set direction

digitalWrite(in2, HIGH);

float analogvalue = analogRead(A0);             // Read analog voltage input

int PWMvalue = map(analogvalue, 0, 1023, 0, 255); // Convert to PWM range

analogWrite(ConA, PWMvalue);                   // Control motor speed

Serial.println("The motor is running in the clockwise direction.");

Serial.print("Digital Value = ");

Serial.print(PWMvalue);

float analogVoltage = (PWMvalue * 5.00) / 255.00;  // Convert to voltage

Serial.print(" Analog Voltage = ");

Serial.println(analogVoltage);

}

void TurnMotorA2() {

digitalWrite(in1, HIGH);   // Reverse direction

digitalWrite(in2, LOW);

float analogvalue = analogRead(A0);

int PWMvalue = map(analogvalue, 0, 1023, 0, 255);

analogWrite(ConA, PWMvalue);

Serial.println("The motor is running in the anticlockwise direction.");

Serial.print("Digital Value = ");

Serial.print(PWMvalue);

float analogVoltage = (PWMvalue * 5.00) / 255.00;

Serial.print(" Analog Voltage = ");

Serial.println(analogVoltage);

}

void loop() {

if (digitalRead(switchrotation) == LOW) {

TurnMotorA1();

}

else if (digitalRead(switchrotation) == HIGH) {

TurnMotorA2();

}

}

MES Lab_9 Expected Outcomes and Alternative Codes

When switchrotation = LOW (Clockwise direction)

| Potentiometer Position | analogRead(A0) | PWMvalue | Approx Voltage (V) | Serial Text |
| --- | --- | --- | --- | --- |
| 0% (min) | 0 | 0 | 0.00 V | The motor is running in the clockwise direction.Digital Value = 0, Analog Voltage = 0.00 |
| 25% | ~256 | ~64 | 1.25 V | ... clockwise direction.Digital Value = 64, Analog Voltage = 1.25 |
| 50% | ~512 | ~127 | 2.49 V | ... clockwise direction.Digital Value = 127, Analog Voltage = 2.49 |
| 75% | ~768 | ~191 | 3.74 V | ... clockwise direction.Digital Value = 191, Analog Voltage = 3.74 |
| 100% (max) | 1023 | 255 | 5.00 V | ... clockwise direction.Digital Value = 255, Analog Voltage = 5.00 |

When switchrotation = HIGH (Anticlockwise direction)

| Potentiometer Position | analogRead(A0) | PWMvalue | Approx Voltage (V) | Serial Text |
| --- | --- | --- | --- | --- |
| 0% (min) | 0 | 0 | 0.00 V | The motor is running in the anticlockwise direction.Digital Value = 0, Analog Voltage = 0.00 |
| 25% | ~256 | ~64 | 1.25 V | ... anticlockwise direction.Digital Value = 64, Analog Voltage = 1.25 |
| 50% | ~512 | ~127 | 2.49 V | ... anticlockwise direction.Digital Value = 127, Analog Voltage = 2.49 |
| 75% | ~768 | ~191 | 3.74 V | ... anticlockwise direction.Digital Value = 191, Analog Voltage = 3.74 |
| 100% (max) | 1023 | 255 | 5.00 V | ... anticlockwise direction.Digital Value = 255, Analog Voltage = 5.00 |