Sample Code

#define RED_PIN 8

#define YELLOW_PIN 10

#define GREEN_PIN 12

//Define the delays for each traffic light color

int red_on = 3000;

int green_on = 3000;

int yellow_blink = 500;

// Declaring the timer function to count 1 ms of delay instead of calling the delay function

int delay_timer (int milliseconds){

int count = 0;

while(1)

{

if(TCNT0 >= 100) // Checking if 1 millisecond has passed

{

TCNT0 = 0;

count++;

if (count == milliseconds) //checking if required milliseconds delay has passed

{

count = 0;

break; // exits the loop

}

}

}

return 0;

}

void setup() {

//Define pins connected to LEDs as outputs

pinMode(RED_PIN, OUTPUT);

pinMode(YELLOW_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

//Set up the Timer0

TCCR0A = 0b00000000;

TCCR0B = 0b00000000; //setting pre-scaler for timer clock

TCNT0 = 0;

}

void loop() {

//to make the green LED turned on

digitalWrite(GREEN_PIN, HIGH);

delay_timer(green_on);

//to make the green LED turned off

digitalWrite(GREEN_PIN, LOW);

//for turning the yellow LED on and off for 4 times

for(int i = 0; i < 6; i = i+1)

{

digitalWrite(YELLOW_PIN, HIGH);

delay_timer(yellow_blink);

digitalWrite(YELLOW_PIN, LOW);

delay_timer(yellow_blink);

}

//to make the red LED turned on

digitalWrite(RED_PIN, HIGH);

delay_timer(red_on);

//to make the red LED turned off

digitalWrite(RED_PIN, LOW);

}

void setup() {

// pin connections for the LED lights

pinMode(8, OUTPUT);

pinMode(10, OUTPUT);

pinMode(12, OUTPUT);

}

void loop() {

// Turning on the voltage at the output pin 8 (for green LED)

digitalWrite(8, HIGH);

delay(3000); // green LED is on for 3 seconds

// Turning off the voltage at output pin 8 (for green LED)

digitalWrite(8, LOW); // green LED is off

// Turning the yellow LED on and off for 4 times

for (int i = 0; i < 4; i = i+1)

{

digitalWrite(10, HIGH);

delay(500); // yellow LED is on for 0.5 seconds

digitalWrite(10, LOW);

delay(500); // yellow LED is off for 0.5 seconds

}

// Turning on the voltage at the output pin 12 (for red LED)

digitalWrite(12, HIGH);

delay(6000); // red LED is on for 6 seconds

// Turning off the voltage at the output pin 12 (for red LED)

digitalWrite(12, LOW); // red LED is off

} // void loop() function ends and repeats